EQ-CH150N

Cylinder Head

| Description | Specifications |
| --- | --- |
| New Thickness/MeasuredAt | 3.5000" (88.9 MM) |
| Minimum Thickness |  |
| Warp Spec. | .004" OVERALL (.102 MM) |
| Chamber Volume (cc) | 49.9-52.9 CC |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | .9040-.9045" (22.962-22.974 MM) |
| Comments |  |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.765-1.795" (44.831-45.593 MM) | 1.765-1.795" (44.831-45.593 MM) |
| Stem Diameter | .311-.312" (7.899-7.925 MM) | .311-.312" (7.899-7.925 MM) |
| Total Length | 4.889-4.924" (124.181-125.070 MM) | 4.917-4.952" (124.892-125.755 MM) |
| Head Diameter | 1.905-1.915" (48.387-48.641 MM) | 1.495-1.505" (37.973-38.227 MM) |
| Angle | 45° | 45° |
| Margin | .031" (.787 MM) | .031" (.787 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | INTEGRAL | INTEGRAL |
| Guide Clearance | .0010-.0027" (.025-.069 MM) | .0010-.0027" (.025-.069 MM) |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .040-.060" (1.016-1.524 MM) | .040-.060" (1.016-1.524 MM) |
| Seat Angle | 44.5° | 44.5° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.967" (49.962 MM) | 1.967" (49.962 MM) |
| Installed Height | 1.640" (41.656 MM) | 1.640" (41.656 MM) |
| Open Test Press | 200# @ 1.216 | 200# @ 1.216 |
| Close Test Press | 80-90# @ 1.640 | 80-90# @ 1.640 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash |  |
| Exhaust Valve Lash |  |
| Lash Adjustment Method | HYDRAULIC |
| Precup Torque Spec. |  |
| Cyl Head Torque | 22, 45, 110 FT/LBS |