Cylinder Head

| Description | Specifications |
| --- | --- |
| New Thickness/MeasuredAt | 4.016” (102.000 MM) |
| Minimum Thickness | 4.008" ( 101.803 MM) |
| Warp Spec. | .004" OVERALL (.102 MM) MAX |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments |  |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.700" MEASURED FROM OUTER SPRING PAD | 1.700" MEASURED FROM OUTER SPRING PAD |
| Stem Diameter | .2744-.2750" (6.970-6.985 MM) | .2734-.2740" (6.945-6.960 MM) |
| Total Length | 3.892-3.915" (98.850-99.450 MM | 3.896-3.919" (98.950-99.550 MM) |
| Head Diameter | 1.4961-1.5079" (38.000-38.300 MM) | 1.2598-1.2717" (32.000-32.300 MM) |
| Angle | 45.5º | 45.5º |
| Margin |  |  |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height |  |  |
| Guide Clearance | .0006-.0019" (.015-.048 MM) | .0016-.0029" (.040-.073 MM) |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .051-.059" (1.300-1.500 MM) | .063-.071" (1.600-1.800 MM) |
| Seat Angle | 45º | 45º |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.807" (45.900 MM) | 1.807" (45.900 MM) |
| Installed Height | 1.331" (33.807 MM) | 1.331" (33.807 MM) |
| Open Test Press | 78-88# @ .984" | 78-88# @ .940" |
| Close Test Press | 40-45# @ 1.331" | 40-45# @ 1.331" |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance | .0008-.0021" (.020-.054 MM) |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .014-.016" HOT (.350-.410 MM) |
| Exhaust Valve Lash | .014-.016" HOT (.350-.410 MM) |
| Lash Adjustment Method |  |
| Precup Torque Spec. |  |
| Cyl Head Torque | 14-17, 51 FT/LBS, LOOSEN, REPEAT 14-17 FT/LBS, +90° |