Cylinder Head

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 5.615" (142.621 MM) reported |
| Minimum Thickness | 5.600" (142.240 MM) |
| Warp Spec. | .002" OVERALL (.050 MM) MAX |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments |  |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.710-1.730" (43.434-43.942 MM) | 1.710-1.730" (43.434-43.942 MM) |
| Stem Diameter | .2154-.2159" (5.470-5.485 MM) | .2151-.2157" (5.465-5.480 MM) |
| Total Length | 4.1835" (106.260 MM) | 4.2024" (106.740 MM) |
| Head Diameter |  |  |
| Angle |  |  |
| Margin | .020" MIN (.500 MM) | .020" MIN (.500 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height |  |  |
| Guide Clearance | .0010-.0024" (.025-.060 MM) | .0012-.0026" (.030-.065 MM) |
| Guide OD |  |  |
| Valve Seal Type |  |  |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .039-.055" (1.000-1.400 MM) | .039-.055" (1.000-1.400 MM) |
| Seat Angle | 45º | 45º |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.9106" (48.530 MM) | 1.9106" (48.530 MM) |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .006-.010" (.152-.254 MM) |
| Exhaust Valve Lash | .010-.014" (.254-.356 MM) |
| Lash Adjustment Method | 2005 UP HYD LIFTERS |
| Precup Torque Spec. |  |
| Cyl Head Torque | 29 FT/LBS, +90º, +90º |