Casting

| Description | Specifications |
| --- | --- |
| New Thickness/MeasuredAt | 4.5850-4.5950" (116.459-116.713 MM) |
| Minimum Thickness | 4.5850" (116.459) |
| Warp Spec. | .002" OVERALL (.051 MM) |
| Chamber Volume (cc) | 2002-2004 is D shaped and 2005-> is kidney shaped |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter |  |
| Comments | * IN THE 2002 MANUAL: EXHAUST FREE LENGTH=1.9610", EXHAUST CLOSED=88-97# @ 1.5795", EXHAUST OPEN=217-237# @ 1.107, INTAKE OPEN=221-234# @ 1.107". |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 2.014-2.058" (51.170-52.270 MM) | 2.071-2.114" (52.610-53.710 MM) |
| Stem Diameter | .2729-.2739" (6.931-6.957 MM) | .2717-.2728" (6.902-9.928 MM) |
| Total Length | 4.4666-4.4965" (113.450-114.210 MM) | 4.5244-4.5543" (114.920-115.680 MM) |
| Head Diameter | 1.9103-1.9205" (48.520-48.780 MM) | 1.4516-1.4618" (36.870-37.130 MM) |
| Angle | 45-45.5º | 45-45.5º |
| Margin |  |  |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .525" (13.335 MM) | .525" (13.335 MM) |
| Guide Clearance | .0008-.0028" (.018-.069 MM) | .0019-.0039" (.047-.098 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .070-.093" (1.750-2.360 MM) | .067-.091" (1.710-2.320 MM) |
| Seat Angle | 44.5-45º | 44.5-45º |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.896" (48.180 MM) | 1.897" (48.200 MM) * |
| Installed Height | 1.579" (40.120 MM) | 1.579" (40.120 MM) (WI/out DAMPER) |
| Open Test Press | 214-234# @ 1.107" | 215-219# @ 1.067" (W/OUT DAMPER) * |
| Close Test Press | 75-83# @ 1.579" | 80-89# @ 1.540" (W/OUT DAMPER) * |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash |  |
| Exhaust Valve Lash |  |
| Lash Adjustment Method |  |
| Precup Torque Spec. |  |
| Cyl Head Torque | 4 STEPS - SEE "NOTES TAB ABOVE" |
| Camshaft Cap Torque | 100 INCH/LBS |