Casting

| Description | Specifications |
| --- | --- |
| New Thickness/Measured At | 3.4650" (88.011 MM) |
| Minimum Thickness | 3.4370"(87.299 MM) ** SEE COMMENTS BOX BELOW |
| Warp Spec. | .008" OVERALL (.203 MM) |
| Chamber Volume (cc) | 52.22 - 58.22 CC |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | .9040-.9045" (22.962-22.974 MM) |
| Comments | * REPORTED INSTALLED VALVE STEM HEIGHT 1.9900-2.0100" (50.546-51.054 MM) ON CYLINDER HEAD CASTING ** REPORTED TO WORK FINE |

Valve

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.7750-1.8050" (45.085-45.847 MM) * | 1.7750-1.8050" (45.085-45.847 MM) * |
| Stem Diameter | .3110-.3120" (7.899-7.925 MM) | .3110-.3120" (7.899-7.925 MM) |
| Total Length | 4.8220-4.8370" (122.479-122.86 MM) | 4.8307-4.8520" (122.86-123.241 MM) |
| Head Diameter | 1.9050-1.9150" (48.387-48.641 MM) | 1.4950-1.5050" (37.973-38.227 MM) |
| Angle | 46° | 46° |
| Margin | .031" MIN. (.787 MM) | .031" MIN. (.787 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | INTEGRAL | INTEGRAL |
| Guide Clearance | .0010-.0030" (.025-.076 MM) | .0012-.0030" (.030-.076 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .040-.060" (1.016-1.524 MM) | .040-.060" (1.016-1.524 MM) |
| Seat Angle | 30° / 44.5° / 65° | 30° / 44.5° / 65° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 1.8760" APPROX (47.65 MM) | 1.8760" APPROX (47.65 MM) |
| Installed Height | 1.6400" (41.656 MM) | 1.6400" (41.656 MM) |
| Open Test Press | 202-218# @1.2160" | 202-218# @1.2160" |
| Close Test Press | 71-79# @ 1.6400" | 71-79# @ 1.6400" |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | 1.6:1 RATIO |
| Exhaust Valve Lash | 1.6:1 RATIO |
| Lash Adjustment Method | HYDRAULIC |
| Precup Torque Spec. |  |
| Cyl Head Torque | 22, 45, 110 FT/LBS, EXCEPT BOLT #11=100 FT/LBS |