Casting

| Description | Specifications |
| --- | --- |
| New Thickness/MeasuredAt | 2.8900-2.8950" (73.406 TO 73.533 MM) *** |
| Minimum Thickness | 2.8800" (73.152 MM) |
| Warp Spec. | .006" OVERALL (.152 MM) |
| Chamber Volume (cc) | 1990-94 60 CC'S 1995-2000 48 CC'S, Marnel 43 cc's |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | .8742-.8755" (22.205 TO 22.238 MM) |
| Comments | ** ALWAYS USE NEW BOLTS *** 2.9130-2.9170" REPORTED ON NON FACTORY HEADS CHAMBER SIZE OK |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.7350-1.7450" (44.069 TO 44.323 MM) | 1.7350-1.7450" (44.069 TO 44.323 MM) |
| Stem Diameter | .3159-.3167" (8.024 TO 8.044 MM) | .3149-.3156" (7.998 TO 8.016 MM) |
| Total Length |  |  |
| Head Diameter | 1.710" (43.434 MM) | 1.358" (34.493 MM) |
| Angle | 44° | 44° |
| Margin | .0310" (.787 MM) | .0310" (.787 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .650-.660" (16.51 TO 16.764 MM) | .650-.660" (16.51 TO 16.764 MM) |
| Guide Clearance | .0008-.0025" (.02 TO .064 MM) | .0018-.0035" (.046 TO .089 MM) |
| Guide OD |  |  |
| Valve Seal Type | POSITIVE | POSITIVE |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .060-.079" (1.524 TO 2.007 MM) | .060-.079" (1.524 TO 2.007 MM) |
| Seat Angle | 45° | 45° |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 2.140" (54.356 MM) | 2.140" (54.356 MM) |
| Installed Height | 1.578-1.609" (40.081 TO 40.869 MM) | 1.578-1.609" (40.081 TO 40.869 MM) |
| Open Test Press | 162-175#s @1.180" | 162-175#s @1.180" |
| Close Test Press | 76-86# @ 1.585" SEE TB 1316 | 76-86#s @ 1.585" SEE TB 1366 |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .090-.170" DRY (2.286 TO 4.318 MM) |
| Exhaust Valve Lash | .090-.170" DRY (2.286 TO 4.318 MM) |
| Lash Adjustment Method | HYD |
| Precup Torque Spec. |  |
| Cyl Head Torque | 44, 55 FT/LBS, +80-85° --- |