Casting

| Description | Specifications |
| --- | --- |
| NewThickness/ MeasuredAt |  |
| Minimum Thickness |  |
| Warp Spec. | .003" OVERALL (.076 MM) |
| Chamber Volume (cc) |  |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | .9035-.9040" (22.949-22.962 MM) |
| Comments | *1993-02 SEE INNER SPRING FOR SPECS. ** 1998-02 HEAD DIAMETER INTAKE=1.916", EXHAUST=1.624" |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 1.7830-1.8500" (45.288-46.99 MM) | 1.7830-1.8500" (45.288-46.99 MM) |
| Stem Diameter | .3110-.3120" (7.899-7.925 MM) | .3110-.3120" (7.899-7.925 MM) |
| Total Length | 4.893-4.918" (124.282-124.917 MM) | 4.907-4.932" (124.638-125.273 MM) |
| Head Diameter | 1.880" (47.752 MM) ** | 1.600" (40.64 MM) ** |
| Angle | 43.5° | 43.5° |
| Margin | .047" MIN. (1.194 MM) | .047" MIN. (1.194 MM) |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | INTEGRAL | INTEGRAL |
| Guide Clearance | .0010-.0030" (.025-.076 MM) | .0010-.0030" (.025-.076 MM) |
| Guide OD |  |  |
| Valve Seal Type | POS | POS |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .040-.060" (1.016-1.524 MM) | .060-.080" (1.524-2.032 MM) |
| Seat Angle | 44.5° | 44.5° |
| Seat Outside Diameter | DO NOT REMOVE SHROUDING | DO NOT REMOVE SHROUDING |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter | .3130-.3140" (7.95-7.976 MM) | .3130-.3140" (7.95-7.976 MM) |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 2.000" (50.8 MM) * | 1.810" (45.974 MM) * |
| Installed Height | 1.630-1.690" (41.402-42.926 MM) * | 1.450-1.520" (36.83-38.608 MM) * |
| Open Test Press | 170-184# @1.312 * | 180-194# @1.062 * |
| Close Test Press | 78-88# @ 1.687 * | 80-90# @ 1.484 * |
| Inner Spring | Intake | Exhaust |
| Free Length | 1.967" (49.962 MM) * | 1.967" (49.962 MM) * |
| Installed Height | 1.640" (41.656 MM) * | 1.640" (41.656 MM) * |
| Open Test Press | 200# @ 1.212 * | 200# @ 1.212 * |
| Close Test Press | 85# @ 1.640 * | 85# @ 1.640 * |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | .060-.210" DRY (1.524-5.334 MM) |
| Exhaust Valve Lash | .060-.210" DRY (1.524-5.334 MM) |
| Lash Adjustment Method | NONE |
| Precup Torque Spec. |  |
| Cyl Head Torque | 50, 105 FT/LBS |