Casting

| Description | Specifications |
| --- | --- |
| New Thickness /Measured At |  |
| Minimum Thickness |  |
| Warp Spec. |  |
| Chamber Volume (cc) | 51.5-52.5 CC'S |
| Surface Finish |  |
| OHC Camshaft Bore Diameter |  |
| Injector Tip Protrusion |  |
| Lifter Bore Diameter | LIFTER TO BORE CLEARANCE .0007-.0027" (.018-.069 MM) |
| Comments | CAM CAPS 89 IN/LBS * LIFTER BLEED DOWN RATE 5-25 SECONDS |

Valve

Collapse

| Valve Information | Intake | Exhaust |
| --- | --- | --- |
| Installed Stem Height | 2.2300-2.2400" (56.642-56.896 MM) | 2.2300-2.2400" (56.642-56.896 MM) |
| Stem Diameter | .2742-.2750" (6.965-6.995 MM) | .2732-.2740" (6.940-6.970 MM) |
| Total Length |  |  |
| Head Diameter | 1.750" (44.50 MM) | 1.410" (36.0 MM ) |
| Angle | 45.25'-45.75' | 45.25'-45.75' |
| Margin |  |  |
| Recession |  |  |
| Valve Guide Information | Intake | Exhaust |
| Guide Height | .600-.605" (15.200 MM) | .600-.605" (15.200 MM) |
| Guide Clearance | .00078-.00311" (.020-.079 MM) | .0017-.0040" (.045-.104 MM) |
| Guide OD |  |  |
| Valve Seal Type | POS TOP HAT | POS TOP HAT |
| Valve Seat Information | Intake | Exhaust |
| Seat Width | .075-.083" (1.900-2.100 MM) | .075-.083" (1.900-2.100 MM) |
| Seat Angle | 44.51' | 45.01' |
| Seat Outside Diameter |  |  |
| Seat Inside Diameter |  |  |
| Seat Insert Height |  |  |
| Seat Insert Bore Diameter |  |  |
| Seat Insert Bore Depth |  |  |

Valve Spring

Collapse

| Outer or Single Spring | Intake | Exhaust |
| --- | --- | --- |
| Free Length | 2.100" (53.370 MM) | 2.100" (53.370 MM) |
| Installed Height |  |  |
| Open Test Press | 162-180# @ 1.1340" (28.80 MM) | 162-180# @ 1.1340" (28.80 MM) |
| Close Test Press | 64-72# @ 1.6756" 42.56 MM) | 64-72# @ 1.6756" 42.56 MM) |
| Inner Spring | Intake | Exhaust |
| Free Length |  |  |
| Installed Height |  |  |
| Open Test Press |  |  |
| Close Test Press |  |  |

Rocker Shaft

Collapse

| Description | Specifications |
| --- | --- |
| Shaft Diameter |  |
| Bore Diameter |  |
| Oil Clearance |  |
| Rock Arm Ratio |  |

Installation

Collapse

| Description | Specifications |
| --- | --- |
| Intake Valve Lash | DRY .045-.085" * |
| Exhaust Valve Lash | DRY .045-.085" |
| Lash Adjustment Method | ROCKER RATIO 1.8:1 |
| Precup Torque Spec. |  |
| Cyl Head Torque | 30 FT/LBS, +90°, LOOSEN ALL, 30 FT/LBS, +90°, +90° |