WF920 Series – Combination Winter Front and Bug Screen

# INSTALLATION INSTRUCTIONS

IMPORTANT NOTE:  THE BUG SCREEN MUST BE USED AS THE TEMPLATE

Before installing Stick A Stud TM *D* fasteners, please note:

- Clean area with isopropyl alcohol swab (supplied)

- Surface must be completely dry, wax free with no cleaning fluid present.

- For best results, install at temperatures over 60oF (16oC). Use a hair dryer to

warm up the surface and the Stick A Stud™ *D* if necessary.

- The base of the Stick A Stud™ *D* can be shaped by bending or folding along the indented lines

FOR CONCAVE OR CONVEX SURFACES              FOR ANGLED SURFACES

- In some instances the Stick A Stud TM *D*  fastener’s base may require trimming.  Simply cut as required

INSTALLATION:

- Snap all Stick A Stud TM *D*  fasteners to the bug screen while ensuring the protective backings remain intact.

- Position the bug screen on the vehicle and note where each Stick A Stud TM *D*  fastener will be located to ensure there will be maximum surface contact at each installation point.

- Temporarily fasten the bug screen with masking tape to the vehicle.

- Remove protective backing from a corner Stick A Stud TM *D*  . Avoid touching adhesive.

- Apply firm pressure for 10 seconds.

- Apply the next Stick A Stud TM *D*   fastener ensuring that no stress is applied to other fasteners when installing the bug screen.  CAUTION: A MINIMAL AMOUNT OF STRESS ON THE BUG SCREEN FROM SIDE TO SIDE AND TOP TO BOTTOM IS RECOMMENDED.  EXCESSIVE STRESS WILL CAUSE IMPROPER INSTALLATION.

- Attach remaining Stick A Stud TM *D*  fasteners along top and bottom.

- Carefully remove bug screen and again apply firm pressure to each Stick A StudTM *D* fastener.  INSTANT BOND CAN BE INCREASED BY USING HAIR DRYER ON HIGH FOR 2-3 MINUTES ON EACH INSTALLED STUD.

- Allow a minimum of 24 hours before use.  See chart on reverse.

BOND STRENGTH

| TIME | BOND |
| --- | --- |
| 20 minutes | 50% |
| 60 minutes | 75% |
| 24 hours | 90% |
| 72 hours | 100% |

Additional Stick A Stud TM *D*  fasteners are available; part number HK104

They are also available in white; part number HK108

Kits with studs and screws are also available; part number HK103

NOTES

- Models requiring adhesive velcro installation:

BEFORE APPLYING VELCRO:

- Ensure the area where velcro strips are to be attached is clean and dry.

- Remove protective backing

- Secure in place  by pressing firmly on velcro

Step 2-B: Attach adhesive backed velcro strip to the underside of the grill frame to line up with velcro on bug screen.  On some vehicles the adhesive backed velcro strips require mounting on the front of the grill frame or under the hood on the radiator frame.

- Models requiring attachment to grill face and bumper for installation:

WF921-24, WF924-6, WF926-7, WF927-3, WF929-34, WF929-48, GS901-24, GS904-6, GS906-7, GS907-3, GS909-34, GS909-48

- Models requiring attachment to the top of the grill frame under the hood and to the bumper:   WF925-7, WF927-32, WF927-33, WF929-45, GS905-7, GS907-32, GS907-33, GS909-45

- Models requiring attachment to the hood:   WF921-17,WF921-19, WF921-20, WF921-21, WF923-4, WF925-42, WF929-281, WF929-46, GS901-17, GS901-19,

GS901-20, ,  GS901-21, GS903-4, GS905-42, GS909-281, GS909-46

- Models requiring attachment to grill face and to top of the grill frame under the hood:   WF921-15, WF921-16, WF922-9, WF924-11, WF924-12, WF924-7, WF924-8, WF925-6, WF925-71, WF926-81, WF926-9, WF929-22, WF929-26, WF929-361,

WF929-42, WF929-43, GS901-15, GS901-16, GS902-9, GS904-11, GS904-12, GS904-7, GS904-8, GS905-6, GS905-71, GS906-81, GS906-9, GS909-22, GS909-26, GS909-361, GS909-42, GS909-43

- On some models there are more domes than are required for installation.  Align unit with grill to determine which domes are required.

Should you require further information on installing this product, please contact our

customer service department at:

Fia Inc.  Winnipeg, Canada  R3L 0N3

Telephone (204) 475-0515  Toll-Free: 1-800-835-0438

Fax: (204) 284-8246  E-Mail: info@gofia.com   Website: www.gofia.com

06-22-12